# Palthope VM0042 test — where every value comes from

Project: PRJ-2026-000001 · Palthope VM0042 pilot · database dmrv_db1 · branch fixes_14/10 · Verra VCS, methodology VM0042 v2.2 Status at the time of writing: Stages 1–13 complete; calculation CALC-2026-000001 approved with 61 VCUs.

This page lists every value entered in the platform for this test and says whether it came from the soil reports, from real project data, or was entered manually as a test value — and why.

| Mark | Meaning |
| --- | --- |
| 🟢 Report | Taken from the Ganesh Consultancy soil reports |
| 🔵 Real | Real project data that is not in the reports (FarmFuture map, farmer phone) |
| 🟡 Derived | Chosen from the reports (e.g. period dates set around the report dates) |
| 🧪 Manual (test) | Not in the reports; entered so the platform can run the full flow |
| ⚙ Methodology / setup | Fixed by Verra, VM0042 or the test setup |

## 1. The two soil reports

Lab: Ganesh Consultancy & Analytical Services, Mysore (ISO 17025). Customer: Varsapradaya Private Limited. Samples drawn by the customer.

| Round | Report | Collected | Samples and organic carbon (OC) |
| --- | --- | --- | --- |
| Baseline (Period 0) | C-11169 … C-11172 | 16-09-2026 | 3 Soil 1.40 % · 2/Weather 2.39 % · Palthope Hub 2.01 % · Palthope Probe 1.74 % |
| Monitoring (Period 1) | C-11543 … C-11546 | 25-09-2026 | P1 1.83 % · P2 1.78 % · H1 1.14 % · H2 2.77 % |

The reports also give pH, EC, available N, P and K. VM0042 does not use these, so they are not entered. Only organic carbon is used.

## 2. Values from the reports (🟢 / 🟡)

| Where in the platform | Value entered | From the report |
| --- | --- | --- |
| Period 0 lab result LT-2026-000001 (P0 S1-1 0-30, V42_OC) | 20.1 g/kg | 🟢 Palthope Hub 2.01 % × 10 |
| Period 0 lab result LT-2026-000005 (P0 S1-2 0-30, V42_OC) | 17.4 g/kg | 🟢 Palthope Probe 1.74 % × 10 |
| Period 0 lab result LT-2026-000009 (P0 S1-3 0-30, V42_OC) | 23.9 g/kg | 🟢 2/Weather 2.39 % × 10 |
| Period 0 lab result LT-2026-000013 (P0 S1-4 0-30, V42_OC) | 14.0 g/kg | 🟢 3 Soil 1.40 % × 10 |
| Period 1 lab result (P1 S1-1 0-30, V42_OC) | 11.4 g/kg | 🟢 H1 1.14 % × 10 |
| Period 1 lab result (P1 S1-2 0-30, V42_OC) | 18.3 g/kg | 🟢 P1 1.83 % × 10 |
| Period 1 lab result (P1 S1-3 0-30, V42_OC) | 27.7 g/kg | 🟢 H2 2.77 % × 10 |
| Period 1 lab result (P1 S1-4 0-30, V42_OC) | 17.8 g/kg | 🟢 P2 1.78 % × 10 |
| Field records Period 0 — "Collected at" | 16-09-2026 10:00 | 🟢 baseline collection date |
| Field records Period 1 — "Collected at" | 25-09-2026 10:00 | 🟢 monitoring collection date |
| PDF report on all 52 Period 0 lab results | Ganesh Consultancy - Soil Test-16-09-2026.pdf | 🟢 the baseline report |
| PDF report on all 52 Period 1 lab results | Varsapradaya Pvt ltd - Soil - 25-09-2026.pdf | 🟢 the monitoring report |
| Sample quantity per field record | 1 kg | 🟢 "Polythene cover / 1 kg" |
| Laboratory method text (V42_OC) | Dry combustion | 🟢 report method GCAS/SOP/7.2/TOC (organic carbon) |
| Monitoring period Period 0 baseline | 01-09-2026 – 20-09-2026 | 🟡 chosen to contain the 16-09-2026 sampling |
| Monitoring period period 1 | 21-09-2026 – 20-09-2027 | 🟡 chosen to contain the 25-09-2026 sampling |
| Project start, crediting start, MRV plan start, participation, agreement | 01-09-2026 | 🟡 chosen to start before the first sampling |

## 3. Values entered manually, stage by stage

### Stage 1–3 · Setup, catalog, methodology

| Value | Entered | Source | Why |
| --- | --- | --- | --- |
| Organizations (DEV, LAB, VVB, BUY, VREG) and 19 users, password Test@12345678 | — | ⚙ setup | test accounts |
| Standard VCS, activity IALM | — | ⚙ Verra | the crediting programme |
| Methodology VM0042 version 2.2, effective 21-10-2025 | — | ⚙ Verra VM0042 v2.2 | the methodology document |
| Calculation module VM0042-V2.2-QA2-QA3 v1.2.0 (16 monitoring rules) | — | ⚙ platform | implements VM0042 |
| Applicability rules A1 ("VCS"), A2 ("IALM") | — | ⚙ methodology | eligibility check |

### Stage 4 · Farmer

| Value | Entered | Source | Why |
| --- | --- | --- | --- |
| Farmer name | srikanth dodla | 🔵 real | owner of the FarmFuture account |
| Phone | 7799661222 | 🔵 real | finds the estate boundaries in FarmFuture |
| Village / district / state | Palthope / sub-karnataka / karnataka | 🔵 / 🧪 | location (district typed as a placeholder) |
| Language / gender | mr / MALE | 🧪 | required fields |
| KYC National ID | ends 3456 | 🧪 | KYC is required to activate the farmer |
| Consent | Personal data processing v1, paper signed | 🧪 | required before activation |
| Agreement | AGR-2026-000001 · PROGRAM_PARTICIPATION · v1 · from 01-09-2026 | 🧪 / 🟡 | carbon rights need a signed agreement |

### Stage 5 · Farms

| Farm | Boundary | Area | Source | Why |
| --- | --- | --- | --- | --- |
| Palthope Estate (FARM-…001, project) | FarmFuture "Palthope estate", whole farm (2 zones merged) | 7.63 ha | 🔵 real | the project farm |
| Palthope Control C1 (FARM-…003) | drawn square, 300 m west of the estate | 2.89 ha | 🧪 | VM0042 needs 3 control farms; the reports have none |
| Palthope Control C2 (FARM-…004) | drawn square, 300 m east | 2.89 ha | 🧪 | same |
| Palthope Control C3 (FARM-…005) | drawn square, 300 m south | 2.89 ha | 🧪 | same |
| Survey references, valid from, ownership 100 % | Sy No 45/2, 46/1–46/3 · 01-01-2020 | — | 🧪 | ownership must be recorded and verified |

The FarmFuture field "Palthope Bison Field" was first tried as control C1, but FarmFuture maps it as a 0.18 ha triangle in central Bengaluru (196 km away). That farm (FARM-…002) was made inactive and replaced by the drawn square.

### Stage 6 · Project

| Value | Entered | Source | Why |
| --- | --- | --- | --- |
| Name / type / region | Palthope VM0042 pilot · Agricultural land management · Palthope | 🧪 | project record |
| Crediting period | 01-09-2026 – 31-08-2036 | 🧪 / 🟡 | 10 years from the project start |
| Baseline period | 01-09-2023 – 31-08-2026 | 🧪 | the 3 years before the project |
| Baseline practices | High synthetic N, diesel machinery, burning of coffee prunings | 🧪 | describes the old practice |
| Carbon rights | farmer 100 %, from 01-09-2026 | 🧪 | the farmer owns the carbon rights |

### Stage 8 · MRV plan, periods, strata

| Value | Entered | Source | Why |
| --- | --- | --- | --- |
| MRV plan | Annual sampling · 01-09-2026 – 31-08-2036 | 🧪 / 🟡 | monitoring window |
| Periods | see section 2 | 🟡 | contain the report dates |
| Strata | S1 = Palthope Estate; C1–C3 = control farms, each linked to S1 | 🧪 | project area vs control sites |
| Soil texture · soil group · climate | sandy clay loam · Red lateritic · Tropical wet | 🧪 | control farms must match the project (VM0042 Table 7); the reports give no texture |
| Annual rainfall | S1 2200 · C1 2150 · C2 2200 · C3 2250 mm | 🧪 | required; above 1,000 mm the wet-climate N₂O factors apply |

### Stages 10 and 12 · Sampling and laboratory (both periods)

| Value | Entered | Source | Why |
| --- | --- | --- | --- |
| Sampling design | 4 points on the estate (one per report sample), 3 per control farm; stratified random; 0–50 cm; 10 m apart | 🟡 / 🧪 | 4 report samples per round; VM0042 needs at least 3 per stratum |
| GPS of each field record | the planned point's coordinates | ⚙ platform | generated by the platform |
| Probe diameter / cores | 21.5 mm / 4 | 🧪 | needed to turn soil mass into tonnes per hectare (VM0042 Eq. 3) |
| Depth layers | 0–30 cm and 30–50 cm | 🧪 | VM0042 needs two layers; the reports have one sample and no depth |
| OC of the 30–50 cm layer (estate) | 0.6 × the top layer (P0: 12.1, 10.4, 14.3, 8.4 · P1: 6.8, 11.0, 16.6, 10.7 g/kg) | 🧪 | second layer not in the reports |
| Soil mass of every sample | 285 g (0–30) · 205 g (30–50) | 🧪 | the reports give no soil mass or bulk density |
| OC of the control farms | P0 ≈ 18.8–19.3 g/kg (top), 11.3–11.6 (bottom); P1 ≈ 18.3–18.8 (top), 11.0–11.3 (bottom) | 🧪 | the reports cover no control farms |
| Sample labels, seals, accession numbers | P0-/P1-<key>-A/B · SEAL-… · LAB-<number> | 🧪 | chain of custody |
| Analysed at | the date of entry | ⚙ platform | must be after the lab receipt (the report's own analysis dates are earlier) |
| Soil-mass method text | Oven-dried at 105 C, sieved < 2 mm | 🧪 | method of the (test) soil mass |

### Stages 11 and 12.14 · Monitoring data

| Value | Entered | Source | Why |
| --- | --- | --- | --- |
| Risk rating (V42_NPR), both periods | 12 % | ⚙ Verra minimum | sets the buffer |
| Synthetic fertiliser N | 9000 / 9200 / 9400 kg (2023–25) → 4500 kg (project year) | 🧪 | credits come from the cut in emissions |
| Organic fertiliser N | 2000 ×3 → 3000 kg | 🧪 | more compost under the new practice |
| Diesel | 15000 ×3 → 7000 L | 🧪 | less machinery |
| Coffee prunings burned | 60000 / 62000 / 64000 → 0 kg dry matter | 🧪 | prunings mulched instead of burned |
| Compost bought from outside | 20 t at 0.15 t C/t | 🧪 | tests the leakage deduction (VM0042 Eq. 33) |
| Shade trees removed | NO | 🧪 | VM0042 may ignore trees only if none are cut |

## 4. Result (CALC-2026-000001, approved)

| Output | Value | Driven by |
| --- | --- | --- |
| Estate soil carbon, Period 0 → 1 | 36.99 → 36.92 t C/ha | 🟢 the real report values |
| Control soil carbon, Period 0 → 1 | 37.35 → 36.34 t C/ha | 🧪 |
| Sampling uncertainty | 100 % → soil change counted as 0 | 🟢 real values: rounds 9 days apart, spots 1.14–2.77 % OC |
| Emission reductions | 62.43 t CO₂e (diesel 23.09 · fertiliser N₂O 34.67 · burning CH₄ 3.75 + N₂O 0.92) | 🧪 farm-activity values |
| Compost leakage | − 1.32 t CO₂e | 🧪 |
| VCU_TOTAL | 61 | 62.43 − 1.32 = 61.11, rounded down |

What this means: the run proves the full platform flow with the real laboratory numbers. Because the two report rounds are only 9 days apart, the estate's soil carbon does not change measurably and the platform correctly gives no soil credit; the 61 credits come from the farm-activity test values. This is therefore not a carbon estimate for Palthope. A real estimate needs a second sampling about a year later, real control farms, soil mass (or bulk density) per depth layer, and the estate's real records.

## 5. Deviations noticed during the run

| Item | Effect |
| --- | --- |
| Field record SP-2026-000004 (Period 0, S1-1) saved with 2 cores instead of 4 | estate Period 0 soil carbon 36.99 instead of 36.98; credits unchanged (61) |
| Field record SP-2026-000010 (Period 0, C1-3) has no sample quantity | none (not used by the calculation) |
| Carbon-rights reference saved as "Carbon rights clause of AGR-2026" (number cut off) | none (free text) |
| A second agreement AGR-2026-000002 left in Draft | none (AGR-2026-000001 is the signed one) |
| FarmFuture "Palthope Bison Field" mapped in Bengaluru | replaced by a drawn control farm; FarmFuture mapping should be corrected |
| Period 1 named "period 1" (lower case) | none; shows as "Period 2 · period 1" on finance pages |